*********************************************************************

** API specification for PL2303G

**

** ver 1.2

**

** 05/04/2020

*********************************************************************

.

- int PL2303G_GetCOMPort_Address (char *szCOMPortAddress)

Parameters

szCOMPortAddress  [out]  Return PL2303 com port address. Ex: COM3

Returns

Zero indicates success.

Nonzero indicates failure.

- int PL2303G_Open_Port (LPVOID szCOMPortAddress, HANDLE *hdrv)

Parameters

szCOMPortAddress  [in] Com port address, ex: COM3

hdrv      [out]  Returns a handle that can be used to access the com port

Returns

Zero indicates success.

Nonzero indicates failure.

- int PL2303G_Close_Port (HANDLE hdrv)

Parameters

hdrv      [in]  A valid handle to an open com port.

Returns

Zero indicates success.

Nonzero indicates failure.

- int PL2303G_Read_OTP_EEPROM_ID (HANDLE hdrv, BYTE *buff, BYTE mode_Select)

Read PL2303HXD OTP.

Parameters

hdrv           [in] Handle to the communication device.

buff           [out]  PL2303HXD OTP buffer.

mode_Select    [in] modeSelect: 0:OTP,

1:EEPROM

Returns

Zero indicates success.

Nonzero indicates failure.

- int PL2303G_GPIO_A_CNT_ENABLE (HANDLE hdrv, __int8 GPIO_Num, BOOL Enable)

PL2303G  has  8  software controllable “General Purpose Input Output A” control signal pin, you can use this API to control the I/O control enable

Parameters

hdrv           [in] Handle to the communication device.

GPIO_Num     [in] The GPIO number, the range is 0 ~7.

Enable         [in] True is enable GPA pin input/output control respectively, false keep GPA pin settings in OTP

Returns

Zero indicates success.

Nonzero indicates failure.

- int PL2303G_GPIO_B_CNT_ENABLE (HANDLE hdrv, __int8 GPIO_Num, BOOL Enable)

PL2303G  has  8  software controllable “General Purpose Input Output B” control signal pin, you can use this API to control the I/O control enable

Parameters

hdrv           [in] Handle to the communication device.

GPIO_Num     [in] The GPIO number, the range is 0 ~7.

Enable         [in] True is enable GPB pin input/output control respectively, false keep GPB pin settings in OTP.

Returns

Zero indicates success.

Nonzero indicates failure.

- int PL2303G_GPIO_A_OUT_ENABLE (HANDLE hdrv, __int8 GPIO_Num, BOOL Enable)

PL2303G  has 8 software controllable “General Purpose Input Output A” control signal pin, you can use this API to control the direction of these pin.

Parameters

hdrv           [in] Handle to the communication device.

GPIO_Num     [in] The GPIO number, the range is 0 ~3.

Enable         [in] True is Output mode, false is input mode.

Returns

Zero indicates success.

Nonzero indicates failure.

- int PL2303G_GPIO_B_OUT_ENABLE (HANDLE hdrv, __int8 GPIO_Num, BOOL Enable)

PL2303G  has 8 software controllable “General Purpose Input Output B” control signal pin, you can use this API to control the direction of these pin.

Parameters

hdrv           [in] Handle to the communication device.

GPIO_Num     [in] The GPIO number, the range is 0 ~3.

Enable         [in] True is Output mode, false is input mode.

Returns

Zero indicates success.

Nonzero indicates failure.

- int PL2303G_GPIO_A_SetValue (HANDLE hdrv, __int8 GPIO_Num, BYTE val)

PL2303G  has 8 software controllable “General Purpose Input Output A” control signal pin, you can use this API to control the status of these pin when the direction is output mode.

Parameters

hdrv           [in] Handle to the communication device.

GPIO_Num     [in] The GPIO number, the range is 0 ~7.

val            [in] “1” is High level, 0 is low level.

Returns

Zero indicates success.

Nonzero indicates failure.

- int PL2303G_GPIO_B_SetValue (HANDLE hdrv, __int8 GPIO_Num, BYTE val)

PL2303G  has 8 software controllable “General Purpose Input Outpu B” control signal pin, you can use this API to control the status of these pin when the direction is output mode.

Parameters

hdrv           [in] Handle to the communication device.

GPIO_Num     [in] The GPIO number, the range is 0 ~7.

val            [in] “1” is High level, 0 is low level.

Returns

Zero indicates success.

Nonzero indicates failure.

- int PL2303G_GPIO_A_GetValue (HANDLE hdrv, __int8 GPIO_Num, BYTE *val)

PL2303G  has 8 software controllable “General Purpose Input Output A” control signal pin, you can use this API to get the status of these pin when the direction is input mode.

Parameters

hdrv           [in] Handle to the communication device.

GPIO_Num     [in] The GPIO number, the range is 0 ~8.

val            [out] “1” is High level, 0 is low level.

Returns

Zero indicates success.

Nonzero indicates failure.

- int PL2303G_GPIO_B_GetValue (HANDLE hdrv, __int8 GPIO_Num, BYTE *val)

PL2303G  has 8 software controllable “General Purpose Input Output B” control signal pin, you can use this API to get the status of these pin when the direction is input mode.

Parameters

hdrv           [in] Handle to the communication device.

GPIO_Num     [in] The GPIO number, the range is 0 ~8.

val            [out] “1” is High level, 0 is low level.

Returns

Zero indicates success.

Nonzero indicates failure.

- int PL2303G_Read_OTP_EEPROM_ID (HANDLE hdrv, BYTE *data_buff, BYTE *modeSelect)

PL2303G: Read OTP / EEPROM data.

Parameters

hdrv           [in] Handle to the communication device.

data_buff     [in] Point to data buffer.

modeSelect    [in] “0” is OTP, 1 is EEPROM.

Returns

Zero indicates success.

Nonzero indicates failure.

- int PL2303G_Write_OTP_AB (HANDLE hdrv, BYTE *data_buff)

PL2303G: Read OTP / EEPROM data.

Parameters

hdrv           [in] Handle to the communication device.

data_buff     [in] Point to data buffer.

Returns

Zero indicates success.

Nonzero indicates failure.

- int PL2303G_Write_EEPROM_A (HANDLE hdrv, BYTE *data_buff)

PL2303G: Pre-Write EEPROM data.

Parameters

hdrv           [in] Handle to the communication device.

data_buff     [in] Point to data buffer.

Returns

Zero indicates success.

Nonzero indicates failure.

- int PL2303G_Write_EEPROM_B (HANDLE hdrv, BYTE *data_buff)

PL2303G: Write EEPROM data. (After PL2303G_Write_EEPROM_A   API)

Parameters

hdrv           [in] Handle to the communication device.

data_buff     [in] Point to data buffer.

Returns

Zero indicates success.

Nonzero indicates failure.